You need to pause the print at 3.6mm if you are printing with 0.2mm layer height!!!
And add 2x M3 square nuts.

Parts in the image are printed with PA12-CF15,

But you could print it with ABS or ASA. PLA and PETG not supported or tested.

PA12-CF15					ABS or ASA

Layer height: 0.2mm				Layer height: 0.2mm

Perimeters: 4					Perimeters: 4

Solid layers Top: 5				Solid layers Top: 5

Solid layers Bottom: 5				Solid layers Bottom: 5

Infill: 40%					Infill: 40%

Infill pattern: Grid				Infill pattern: Grid
Nozzle Temp.: 280C				Nozzle Temp.: 255C
Bed Temp.: 100C				Bed Temp.: 90C
Chamber Temp.: 48-50C			Chamber Temp.: 40-45C
Outer wall: 40 mm/s				Outer wall: 45 mm/s

Inner wall 60 mm/s				Inner wall 120 mm/s

Infill 80 mm/s					Infill 140 mm/s

No support needed!!!				No support needed!!!

As you are printing the parts without support. After the print is done, take Knipex Knips and cut out all the supports in red.